CSC59866-E: Bi-weekly Research Progress Update 2

Topic: AI Agents for Distributed Chip Design
Team: Johir Hossain & Linwei Zheng
Date: April 20, 2026

GitHub Repository

- https://github.com/LWZ459/ai-chip-design-agent

- Account created: Yes. Both team members have access.

Research Question (Revised from Week 1 Feedback)

- How can AI agents collaborate in a distributed system to optimize different stages of chip design?

- Justification of Impact: Chip design is a multi-billion dollar industry where manual optimization takes months. If AI agents can collaborate to place macros on a chip and route wires between them, design time may be reduced from weeks to hours. Faster chip design can enable faster, cheaper, and more energy-efficient processors for devices ranging from smartphones to AI data centers.

Input Datasets, Benchmarks, Models, Environments

- ISPD2005 Benchmark Suite: Industry-standard floorplanning benchmarks with 100-500 macros per chip.

- MCNC Benchmark Suite: Smaller benchmarks (10-50 macros) suitable for prototyping.

- Feasibility Check: Both datasets were downloaded and tested locally and in Google Colab. MCNC will be used for initial prototyping.

- Models/Algorithms: AlphaChip (Google DeepMind) for RL-based floorplanning; FD-MAGRPO for multi-agent RL circuit sizing.

- Environment: Custom Gymnasium environment (SimpleChipEnv), grid size 10x10 prototype scaling to 32x32, 5-10 macros initially scaling to 50.

Output and Metrics

- Expected outputs include final macro positions on a grid, coordinate list, and wirelength value.

- Metrics: Wirelength, Congestion, Runtime, Reward, and Sample Efficiency.

- Baseline comparison: Random agent already implemented; learning agents should outperform it.

Coding Updates

- GitHub repository created.

- Google Colab environment set up.

- SimpleChipEnv class implemented.

- Random agent implemented and tested.

- Prototype executed successfully.

- Prototype result: 5 macros placed in 5 steps with total reward of -25.

1.  Environment Setup

!pip install gymnasium numpy matplotlib

import numpy as np

print("Setup complete!")

print(f"NumPy version: {np.__version__}")

2. Simple Environment Class

class SimpleChipEnv:

def __init__(self, grid_size=10):

self.grid_size = grid_size

self.grid = np.zeros((grid_size, grid_size))

self.macros_placed = []

def reset(self):

self.grid = np.zeros((self.grid_size, self.grid_size))

self.macros_placed = []

return self.grid

def step(self, action):

x, y = action

if self.grid[x, y] == 0:

self.grid[x, y] = 1

self.macros_placed.append((x, y))

reward = -len(self.macros_placed)  # Simple reward

done = len(self.macros_placed) >= 5

return self.grid, reward, done

else:

return self.grid, -10, False

3. Random Agent

def random_action(env):

x = np.random.randint(0, env.grid_size)

y = np.random.randint(0, env.grid_size)

return (x, y)

Report Writing Updates

- Project abstract started using Overleaf LaTeX template.

- Methodology outlined: environment design, random baseline, tabular Q-learning, Deep Q-Network, and multi-agent extension.

Plan for Running Code

- Google Colab will be used for experiments due to free GPU access, easy sharing, and minimal setup.

- Python files will be stored in GitHub and imported into Colab notebooks.

Timeline

- Week 2: Random agent prototype (Completed).

- Week 3: Wirelength reward function and Tabular Q-learning.

- Week 4: Deep Q-Network and comparison against random baseline.

- Week 5: Multi-agent extension.

Challenges and Solutions

- Neural networks are new: complete introductory tutorials.

- PyTorch syntax unfamiliar: follow beginner tutorials.

- Partner communication needs improvement: weekly Zoom meetings scheduled Mondays at 5 PM.

Individual Tasks (Week 2)

- Johir Hossain: Repository setup, environment implementation, random agent, dataset downloads, weekly update.

- Linwei Zheng: Review code, draft wirelength pseudocode, study Gymnasium tutorials.

Joint Group Tasks

- Discussed MDP formulation.

- Agreed to start with single-agent design.

- Agreed to use Manhattan distance for wirelength.

Next Week Goals (Week 3)

- Implement wirelength function.

- Add wirelength to reward.

- Run random agent with new reward.

- Start tabular Q-learning.

- Submit next progress update.

Codes we can try:

- AlphaChip: https://github.com/google-research/circuit_training